Neuron Data Reader Runtime API

Documentation

By Yuanhui He

22.12.2014

NeuronDataReader b11

Noitom Technology Co., Ltd.

This document illustrates how user can use NeuronDataReader library and apply the bone data received by the library.

Contents

Neuron Data Reader Runtime API	1

Documentation	1

1.	Overview	4

1.1.	NeuronDataReader framework	4

1.1.	Skeleton Data Format	4

1.2.	Command and Parameter sync.	4

1.3.	User Agreement	5

2.	Reference	6

2.1.	Data type definitions	6

2.1.1.	Cross-platform data types	6

2.1.2.	Socket connection status	6

2.1.3.	Data version of stream	6

2.1.4.	Header of BVH data stream	7

2.2.	Callbacks and callback register	8

2.2.1.	Skeleton data callback	8

2.2.2.	Command data callback	8

2.2.3.	Socket status callback	8

2.3.	API reference	10

2.3.1.	BRGetLastErrorMessage	10

2.3.2.	BRRegisterFrameDataCallback	10

2.3.3.	BRRegisterCommandDataCallback	10

2.3.4.	BRRegisterConnectionStatusCallback	10

2.3.5.	BRConnectTo	11

2.3.6.	BRGetConnectionStatus	11

2.3.7.	BRDisconnect	11

2.3.8.	BRStartUDPServiceAt	11

2.3.9.	BRIsUDPServiceRunning	12

2.3.10.	BRStopUDPService	12

2.3.11.	BRRegisterCommandDataCallback	12

2.3.12.	BRCommandFetchAvatarDataFromServer	12

2.3.13.	BRCommandFetchAvatarDataFromServer	12

2.3.14.	BRRegisterAutoSyncParmeter	12

2.3.15.	BRUnregisterAutoSyncParmeter	12

3.	Illustration	14

4.	Known Bugs	25

Appendix A: Skeleton data sequence in array	26

Appendix B: BVH header template	27

Appendix C: Skeleton graph	36

Revision history	37

- Overview

  - NeuronDataReader framework

The Axis Neuron software of Noitom can stream BVH motion data through TCP/IP or UDP protocol. The NeuronDataReader plugin(API library) can provide convenience for user to receive and use the BVH data stream or sync parameters by commands with server.

The structure of NeuronDataReader library is shown below.

Fig. 1-1 NeuronDataReader Overview

As most other libraries, NeuronDataReader provides various C/C++ functions for users to interact with the library.

  - Skeleton Data Format

The action data, with BVH format, is output by callback. All information of the skeleton data, such as prefix, displacements settings, etc. are included in a BvhDataHeader parameter. The sequence of bone data in the float data array is shown in Appendix A. Appendix B is showing sample BVH header data, for reference in live data stream.

  - Command and Parameter sync.

NeuronDataReader library are mainly used to read and parser data received from server. Considering lots of parameters used at client must be sync with server, some relevant APIs are added. These APIs could register command IDs need to be automatically notified on server.

  - User Agreement

NeuronDataReader uses a callback method to output the data. So prior connecting to the server, a local function must be registered to receive the data. While registering the data-receiving function, the user can pass the Client Object reference into the NeuronDataReader library so that the library can output the Class Object reference along with the data stream during the callback.

The data-processing thread in the NeuronDataReader is a work thread separated from the UI. So the user-registered data-receiving function cannot access the UI elements directly. However, the data or status of the callback function can be saved into a local array or buffer, so that the UI thread can access the local-buffered data in any other place.

There are some commands in NeuronDataReader library used to sync parameters or data with server.

Since C# or Unity cannot call C++ dynamic lib API directly, NeuronDataReader uses a pure C interface.

If the NeuronDataReader lib is used in C/C++ project on Mac platform, a pre-defined symbol “__OS_XUN__” should be included in the pre-process field to import some custom-defined symbols.

- Reference

Some data types, handles, program interfaces of NeuronDataReader lib are listed below.

  - Data type definitions

    - Cross-platform data types

If the NeuronDataReader lib is used in C/C++ project on Mac platform, a pre-defined symbol “__OS_XUN__” should be included in the pre-process field to import some predefined data types.

#ifdef __OS_XUN__

// Mac OS X, Linux or unix like OS data type definetion

typedef unsigned short       UINT16;

typedef unsigned int         UINT32;

typedef unsigned long long   UINT64;

typedef unsigned short       USHORT;

typedef unsigned char        UCHAR;

typedef unsigned char        BYTE;

typedef wchar_t              WCHAR;

#ifdef BVHDATAREADER_EXPORTS

typedef unsigned int         BOOL;

#endif

#define TRUE                 1

#define FALSE                0

#define CALLBACK             // Empty

#else

#define CALLBACK             __stdcall

#endif

    - Socket connection status

The enumerate type below shows the socket connection status: Connected, Connecting, Disconnected.

// Socket connection status

typedef enum _SocketConnectionStatus

{

CS_Connected,            // Socket is connected

CS_Connecting,           // Is trying to connect to server

CS_Disconnected,         // Disconnected from server

}SocketConnectionStatus;

    - Data version of stream

For different versions of NeuronDataReader, the data structure for communication could be changed, both in meaning and structure. Data version is used to be compatible with the data generated by old version of NeuronDataReader.

// Data version

typedef union BvhOutputDataVersion

{

UINT32 _VersionMask;

struct

{

BYTE BuildNumb;      // Build number

BYTE Revision;       // Revision number

BYTE Minor;          // Subversion number

BYTE Major;          // Major version number

};

} BVH_DATA_VER;

    - Header of BVH data stream

// Header format of BVH data

typedef struct _BvhDataHeader

{

UINT16 HeaderToken1;     // Package start token: 0xDDFF

BVH_DATA_VER DataVersion; // Version of community data format. e.g.: 1.0.0.2

UINT32 DataCount;        // Values count, 180 for without disp data

BOOL   WithDisp;         // With/out dispement

BOOL   WithReference;    // With/out reference bone data at first

UINT32 AvatarIndex;      // Avatar index

UCHAR  AvatarName[32];   // Avatar name

UINT32 Reserved1;        // Reserved, only enable this package has 64bytes length

UINT32 Reserved2;        // Reserved

UINT16 HeaderToken2;     // Package end token: 0xEEFF

} BvhDataHeader;

NeuronDataReader library are mainly used to read and parser data received from server. The data stream of every frame includes the BVH header and BVH motion data of float type.

BVH header is a 64 bytes header, including basic information of BVH data: whether the displacement or prefix is included.

BVH motion data is float-type array. If the data includes reference, the first 6 float number is the displacement and rotation of the reference, normally they would all be 0.

If the data includes displacement, every bone would have 6 float number: 3 displacements and 3 rotation. If the data does not include displacement, only the root node would have 6 float number (3 displacements and 3 rotation), other bones would only have 3 rotations. Please reference Appendix A for the bone sequence.

BVH data is organized as a tree structure, please reference the Appendix B for detailed BVH data structure.

  - Callbacks and callback register

NeuronDataReader lib outputs the skeleton data or socket status through callback functions. So related callback handles for NeuronDataReader lib should be registered firstly to receive these data.

    - Skeleton data callback

typedef void (CALLBACK *FrameDataReceived)(void* customedObj, SOCKET_REF sender, BvhDataHeader* header, float* data);

Parameters

customedObj

User defined object.

sender

Connector reference of TCP/IP client as identity.

header

BvhDataHeader type pointer, to output the BVH data format information.

data

Float type array pointer, to output binary data.

Remarks

The related information of the data stream can be obtained from BvhDataHeader.

    - Command data callback

typedef void (CALLBACK *CommandDataReceived)(void* customedObj, SOCKET_REF sender, CommandPack* pack, void* data);

Parameters

customedObj

User defined object.

sender

Connector reference of TCP/IP client as identity.

Pack

A command package send to or received from server.

data

Data pointer of command, related to the specified command type.

Remarks

The related information of the data stream can be obtained from command identity in pack.

    - Socket status callback

typedef void (CALLBACK *SocketStatusChanged)(void* customedObj, SOCKET_REF sender, SocketConnectionStatus status, char* message);

Parameters

customedObj

User defined object.

sender

Connector reference of TCP/IP client as identity.

status

Indicate the status changes of current socket.

message

Status description.

Note: Since the data-processing in the NeuronDataReader is multi-threaded asynchronous, the data-receiving callback function cannot access the UI element directly. If the data need to be used in the UI thread, it is recommended to save the data from the callback function to a local array.

  - API reference

    - BRGetLastErrorMessage

The error information can be acquired by calling 'BRGetLastErrorMessage' once error appear.

BDR_API char* BRGetLastErrorMessage();

Return Values

Return the last error message.

Remarks

The error information can be acquired by calling 'BRGetLastErrorMessage' once error occurred during function callback.

    - BRRegisterFrameDataCallback

Register the BVH data receiving callback handle:

// Register data-receiving callback handle.

BDR_API void BRRegisterFrameDataCallback(void* customedObj, FrameDataReceived handle);

Parameters

customedObj

User defined object.

handle

A function pointer of FrameDataReceived type.

Remarks

The handle of FrameDataReceived type points to the function address of the client.

    - BRRegisterCommandDataCallback

Register the command data receiving callback handle:

// Register data-receiving callback handle.

BDR_API void BRRegisterCommandDataCallback(void* customedObj, CommandDataReceived handle);

Parameters

customedObj

User defined object.

handle

A function pointer of CommandDataReceived type.

Remarks

The handle of CommandDataReceived type points to the function address of the client.

    - BRRegisterConnectionStatusCallback

Register socket status callback Handle:

// Register socket status callback

BDR_API void BRRegisterConnectionStatusCallback(void* customedObj, SocketStatusChanged handle);

Parameters

customedObj

User defined object.

handle

A function pointer.

Remarks

The handle of SocketStatusChanged type points to the function address of the client.

    - BRConnectTo

Connect to the server with given IP address and port:

// Connect to server

BDR_API SOCKET_REFBRConnectTo(char* serverIP, int nPort);

Parameters

serverIP

Server’s IP address.

nPort

Server’s port.

Return Values

If connected successfully, return a handle of socket as its identity ; otherwise NULL is returned.

    - BRGetConnectionStatus

Check connection status. Actually the function has the same output status with the socket callback handle. If the socket status callback handle has already registered, this function is not necessary.

// Check connect status

BDR_API SocketConnectionStatus BRGetConnectionStatus(SOCKET_REF sockRef);

Return Values

Return the connection status of current socket.

    - BRDisconnect

Disconnect from the server. It should be noted that it is necessary to call this function to disconnect from the server before the program exit, otherwise the program cannot exit as it is blocked by the data-receiving thread.

// Disconnect from server

BDR_API void BRDisconnect(SOCKET_REF sockRef);

    - BRStartUDPServiceAt

Since Axis Neuron can output data by TCP/IP or UDP, the NeuronDataReader can read and parser the two socket data types as well. The BRStartUDPServiceAt function is used to start a service to listen and receive data sent from the server.

// Start a UDP service to receive data at 'nPort'

BDR_API SOCKET_REF BRStartUDPServiceAt(int nPort);

    - BRIsUDPServiceRunning

Check if the UDP service is running.

// Check UDP sercie if is running

BDR_API BOOL BRIsUDPServiceRunning(SOCKET_REF sockRef);

    - BRStopUDPService

Stop UDP service.

// Stop UDP service

BDR_API void BRStopUDPService(SOCKET_REF sockRef);

    - BRRegisterCommandDataCallback

Command data is output in a separate channel by registering a command data callback handle.

// Register command data callback

BDR_API void BRRegisterCommandDataCallback(void* customedObj, CommandDataReceived handle);

    - BRCommandFetchAvatarDataFromServer

Send command to get avatar data from server with refered command Id. command data will be received by callback. Return FALSE if any error occurred. UDP service is not supported for now.

BDR_API BOOL BRCommandFetchAvatarDataFromServer(SOCKET_REF sockRef, int avatarIndex, CmdId cmdId);

    - BRCommandFetchAvatarDataFromServer

Send command to get data from server with refered command Id. command data will be received by callback. Return FALSE if any error occured. UDP service is not supported for now.

BDR_API BOOL BRCommandFetchDataFromServer(SOCKET_REF sockRef, CmdId cmdId);

    - BRRegisterAutoSyncParmeter

Register parameter(s) to server for automatically notifying status changed.

BDR_API BOOL BRRegisterAutoSyncParmeter(SOCKET_REF sockRef, CmdId cmdId);

    - BRUnregisterAutoSyncParmeter

Unregister parameter(s) to server for automatically notifying status changed.

BDR_API BOOL BRUnregisterAutoSyncParmeter(SOCKET_REF sockRef, CmdId cmdId);

- Illustration

NeuronDataReader library supports the most popular developing environments such as C/C++/MFC、WPF/C#、Mac Cocoa, and game engine like Unity, Unigine etc.

Bellow will illustrations how library be used in C#.

For windows platform, start Visual Studio 2012, select”New…” in the start page.

Fig. 3-1

Choose Visual C#--> WPF Application in the template list, name the project “demo_cs”:

Fig. 3-2

Click OK, and an empty WPF project is created.

Fig. 3-3 WPF initial project

As the API in NeuronDataReader.dll cannot be accessed directly in C#, so a function import class need to be created to wrap the functions in library.

NeuronDataReader wrap class

/* Copyright: Copyright 2014 Beijing Noitom Technology Ltd. All Rights reserved.

* Pending Patents: PCT/CN2014/085659 PCT/CN2014/071006

*

* Licensed under the Neuron SDK License Beta Version (the “License");

* You may only use the Neuron SDK when in compliance with the License,

* which is provided at the time of installation or download, or which

* otherwise accompanies this software in the form of either an electronic or a hard copy.

*

* Unless required by applicable law or agreed to in writing, the Neuron SDK

* distributed under the License is provided on an "AS IS" BASIS,

* WITHOUT WARRANTIES OR CONDITIONS OF ANY KIND, either express or implied.

* See the License for the specific language governing conditions and

* limitations under the License.

*/

using System;

using System.Linq;

using System.Text;

using System.Runtime.InteropServices;  // For DllImport()

namespace NeuronDataReaderWraper

{

#region Basic data types

/// <summary>

/// Socket connection status

/// </summary>

public enum SocketConnectionStatus

{

CS_Connected,

CS_Connecting,

CS_Disconnected,

};

/// <summary>

/// Data version

/// </summary>

public struct DataVersion

{

public byte BuildNumb;         // Build number

public byte Revision;          // Revision number

public byte Minor;             // Subversion number

public byte Major;             // Major version number

};

/// <summary>

/// Header format of BVH data

/// </summary>

[StructLayout(LayoutKind.Sequential, Pack=1)]

public struct BvhDataHeader

{

public ushort HeaderToken1;    // Package start token: 0xDDFF

public DataVersion DataVersion;// Version of community data format. e.g.: 1.0.0.2

public UInt32 DataCount;       // Values count, 180 for without disp data

public UInt32 bWithDisp;       // With/out dispement

public UInt32 bWithReference;  // With/out reference bone data at first

public UInt32 AvatarIndex;     // Avatar index

[MarshalAs(UnmanagedType.ByValTStr, SizeConst = 32)]

public string AvatarName;      // Avatar name

public UInt32 Reserved1;       // Reserved, only enable this package has 64bytes length

public UInt32 Reserved2;       // Reserved, only enable this package has 64bytes length

public ushort HeaderToken2;    // Package end token: 0xEEFF

};

#endregion

#region Command data types

/// <summary>

/// Command identitys

/// </summary>

public enum CmdId

{

Cmd_BoneSize,                  // Id used to request bone size from server

Cmd_AvatarName,                // Id used to request avatar name from server

Cmd_FaceDirection,             // Id used to request face direction from server

Cmd_DataFrequency,             // Id used to request data sampling frequency from server

Cmd_BvhInheritance,		       // Id used to request bvh inheritance from server

Cmd_AvatarCount,	           // Id used to request avatar count from server

Cmd_CombinationMode,           //

Cmd_RegisterEvent,             //

Cmd_SetAvatarName,             //

};

// Sensor binding combination mode

public enum SensorCombinationModes

{

SC_ArmOnly,              // Left arm or right arm only

SC_UpperBody,            // Upper body, include one arm or both arm, must have chest node

SC_FullBody,             // Full body mode

};

/// <summary>

/// Header format of Command returned from server

/// </summary>

[StructLayout(LayoutKind.Sequential, Pack=1)]

public struct CommandPack

{

public UInt16 Token1;                   // Command start token: 0xAAFF

public UInt32 DataVersion;              // Version of community data format. e.g.: 1.0.0.2

public UInt32 DataLength;               // Package length of command data, by byte.

public UInt32 DataCount;                // Count in data array, related to the specific command.

public CmdId CommandId;                 // Identity of command.

[MarshalAs(UnmanagedType.ByValArray, SizeConst = 40)]

public byte[] CmdParaments;             // Command paraments

public UInt32 Reserved1;                // Reserved, only enable this package has 32bytes length. Maybe used in the future.

public UInt16 Token2;                   // Package end token: 0xBBFF

};

/// <summary>

/// Fetched bone size from server

/// </summary>

[StructLayout(LayoutKind.Sequential, Pack=1)]

public struct CmdResponseBoneSize

{

[MarshalAs(UnmanagedType.ByValTStr, SizeConst = 60)]

public string BoneName;        // Bone name

public float BoneLength;       // Bone length

};

#endregion

#region Callbacks for data output

/// <summary>

/// FrameDataReceived CALLBACK

/// Remarks

///   The related information of the data stream can be obtained from BvhDataHeader.

/// </summary>

/// <param name="customObject">User defined object.</param>

/// <param name="sockRef">Connector reference of TCP/IP client as identity.</param>

/// <param name="bvhDataHeader">A BvhDataHeader type pointer, to output the BVH data format information.</param>

/// <param name="data">Float type array pointer, to output binary data.</param>

[UnmanagedFunctionPointer(CallingConvention.StdCall)]

public delegate void FrameDataReceived(IntPtr customObject, IntPtr sockRef, IntPtr bvhDataHeader, IntPtr data);

/// <summary>

/// Callback for command communication data with TCP/IP server

/// </summary>

/// <param name="customedObj">User defined object.</param>

/// <param name="sockRef">Connector reference of TCP/IP client as identity.</param>

/// <param name="cmdHeader">A CommandHeader type pointer contains command data information.</param>

/// <param name="cmdData">Data pointer of command, related to the specific command.</param>

/// <remark>The related information of the command data can be obtained from CommandHeader. The data content is identified by its command id.</remark>

[UnmanagedFunctionPointer(CallingConvention.StdCall)]

public delegate void CommandDataReceived(IntPtr customedObj, IntPtr sockRef, IntPtr cmdHeader, IntPtr cmdData);

/// <summary>

/// SocketStatusChanged CALLBACK

/// Remarks

///   As convenient, use BRGetConnectionStatus() to get status manually other than register this callback

/// </summary>

/// <param name="customObject">User defined object.</param>

/// <param name="sockRef">Connector reference of TCP/IP client as identity.</param>

/// <param name="bvhDataHeader">Socket connection status</param>

/// <param name="data">Socket connection status description.</param>

[UnmanagedFunctionPointer(CallingConvention.StdCall)]

public delegate void SocketStatusChanged(IntPtr customObject, IntPtr sockRef, SocketConnectionStatus status, [MarshalAs(UnmanagedType.LPStr)]string msg);

#endregion

// API exportor

public class NeuronDataReader

{

#region Importor definition

#if UNITY_IPHONE && !UNITY_EDITOR

private const string ReaderImportor = "__Internal";

#elif _WINDOWS

private const string ReaderImportor = "NeuronDataReader.dll";

#else

private const string ReaderImportor = "NeuronDataReader";

#endif

#endregion

#region Initialize

/// <summary>

/// Register receiving and parsed frame data callback

/// </summary>

/// <param name="customedObj">Client defined object. Can be null</param>

/// <param name="handle">Client defined function.</param>

[DllImport(ReaderImportor, CallingConvention = CallingConvention.Cdecl, CharSet = CharSet.Ansi)]

public static extern void BRRegisterFrameDataCallback(IntPtr customedObj, FrameDataReceived handle);

/// <summary>

///

/// </summary>

/// <returns></returns>

[DllImport(ReaderImportor, CallingConvention = CallingConvention.Cdecl, CharSet = CharSet.Ansi)]

//[return: MarshalAs(UnmanagedType.LPStr)]

private static extern IntPtr BRGetLastErrorMessage();

/// <summary>

/// Call this function to get what error occured in library.

/// </summary>

/// <returns></returns>

public static string strBRGetLastErrorMessage()

{

// Get message pointer

IntPtr ptr = BRGetLastErrorMessage();

// Construct a string from the pointer.

return Marshal.PtrToStringAnsi(ptr);

}

#endregion

#region TCP Service

// Register TCP socket status callback

[DllImport(ReaderImportor, CallingConvention = CallingConvention.Cdecl, CharSet = CharSet.Ansi)]

public static extern void BRRegisterConnectionStatusCallback(IntPtr customedObj, SocketStatusChanged handle);

// Connect to server by TCP/IP

[DllImport(ReaderImportor, CallingConvention = CallingConvention.Cdecl, CharSet = CharSet.Ansi)]

public static extern IntPtr BRConnectTo(string serverIP, int nPort);

// Check TCP connect status

[DllImport(ReaderImportor, CallingConvention = CallingConvention.Cdecl, CharSet = CharSet.Ansi)]

public static extern SocketConnectionStatus BRGetConnectionStatus(IntPtr sockRef);

// Disconnect TCP socket from server

[DllImport(ReaderImportor, CallingConvention = CallingConvention.Cdecl, CharSet = CharSet.Ansi)]

public static extern void BRDisconnect(IntPtr sockRef);

#endregion

#region UPD Service

// Start a UDP service to receive data at 'nPort'

[DllImport(ReaderImportor, CallingConvention = CallingConvention.Cdecl, CharSet = CharSet.Ansi)]

public static extern IntPtr BRStartUDPServiceAt(int nPort);

// Check UDP sercie if is running

[DllImport(ReaderImportor, CallingConvention = CallingConvention.Cdecl, CharSet = CharSet.Ansi)]

public static extern bool BRIsUDPServiceRunning(IntPtr sockRef);

// Stop UDP service

[DllImport(ReaderImportor, CallingConvention = CallingConvention.Cdecl, CharSet = CharSet.Ansi)]

public static extern void BRStopUDPService(IntPtr sockRef);

#endregion

#region Commands API

/// <summary>

/// Register receiving and parsed Cmd data callback

/// </summary>

/// <param name="customedObj">Client defined object. Can be null</param>

/// <param name="handle">Client defined function.</param>

[DllImport(ReaderImportor, CallingConvention = CallingConvention.Cdecl, CharSet = CharSet.Ansi)]

public static extern void BRRegisterCommandDataCallback(IntPtr customedObj, CommandDataReceived handle);

[DllImport(ReaderImportor, CallingConvention = CallingConvention.Cdecl, CharSet = CharSet.Ansi)]

public static extern bool BRRegisterAutoSyncParmeter(IntPtr sockRef, CmdId cmdId);

[DllImport(ReaderImportor, CallingConvention = CallingConvention.Cdecl, CharSet = CharSet.Ansi)]

public static extern bool BRUnregisterAutoSyncParmeter(IntPtr sockRef, CmdId cmdId);

// Check TCP connect status

[DllImport(ReaderImportor, CallingConvention = CallingConvention.Cdecl, CharSet = CharSet.Ansi)]

public static extern bool BRCommandFetchAvatarDataFromServer(IntPtr sockRef, int avatarIndex, CmdId cmdId);

// Check TCP connect status

[DllImport(ReaderImportor, CallingConvention = CallingConvention.Cdecl, CharSet = CharSet.Ansi)]

public static extern bool BRCommandFetchDataFromServer(IntPtr sockRef, CmdId cmdId);

#endregion

}

}

Then add some controllers in the main window, to connect to the server and display the basic information of the received BVH data:

Fir. 3-4 Interface configuration

Copy the NeuronDataReader.dll to the same folder of “*.exe” file, and then run the program. The result is shown below:

Fig. 3-5 Running Result

- Known Bugs

If any bug or issues not figured out in this document, please report to me at: yuanhui.he@noitom.com

Thank you!

Appendix A: Skeleton data sequence in array

Appendix B: BVH header template

HIERARCHY

ROOT Hips

{

OFFSET 0.00 104.19 0.00

CHANNELS 6 Xposition Yposition Zposition Yrotation Xrotation Zrotation

JOINT RightUpLeg

{

OFFSET -11.50 0.00 0.00

CHANNELS 6 Xposition Yposition Zposition Yrotation Xrotation Zrotation

JOINT RightLeg

{

OFFSET 0.00 -48.00 0.00

CHANNELS 6 Xposition Yposition Zposition Yrotation Xrotation Zrotation

JOINT RightFoot

{

OFFSET 0.00 -48.00 0.00

CHANNELS 6 Xposition Yposition Zposition Yrotation Xrotation Zrotation

End Site

{

OFFSET 0.00 -1.81 18.06

}

}

}

}

JOINT LeftUpLeg

{

OFFSET 11.50 0.00 0.00

CHANNELS 6 Xposition Yposition Zposition Yrotation Xrotation Zrotation

JOINT LeftLeg

{

OFFSET 0.00 -48.00 0.00

CHANNELS 6 Xposition Yposition Zposition Yrotation Xrotation Zrotation

JOINT LeftFoot

{

OFFSET 0.00 -48.00 0.00

CHANNELS 6 Xposition Yposition Zposition Yrotation Xrotation Zrotation

End Site

{

OFFSET 0.00 -1.81 18.06

}

}

}

}

JOINT Spine

{

OFFSET 0.00 13.88 0.00

CHANNELS 6 Xposition Yposition Zposition Yrotation Xrotation Zrotation

JOINT Spine1

{

OFFSET 0.00 11.31 0.00

CHANNELS 6 Xposition Yposition Zposition Yrotation Xrotation Zrotation

JOINT Spine2

{

OFFSET 0.00 11.78 0.00

CHANNELS 6 Xposition Yposition Zposition Yrotation Xrotation Zrotation

JOINT Spine3

{

OFFSET 0.00 11.31 0.00

CHANNELS 6 Xposition Yposition Zposition Yrotation Xrotation Zrotation

JOINT Neck

{

OFFSET 0.00 12.09 0.00

CHANNELS 6 Xposition Yposition Zposition Yrotation Xrotation Zrotation

JOINT Head

{

OFFSET 0.00 9.00 0.00

CHANNELS 6 Xposition Yposition Zposition Yrotation Xrotation Zrotation

End Site

{

OFFSET 0.00 18.00 0.00

}

}

}

JOINT RightShoulder

{

OFFSET -3.50 8.06 0.00

CHANNELS 6 Xposition Yposition Zposition Yrotation Xrotation Zrotation

JOINT RightArm

{

OFFSET -17.50 0.00 0.00

CHANNELS 6 Xposition Yposition Zposition Yrotation Xrotation Zrotation

JOINT RightForeArm

{

OFFSET -29.00 0.00 0.00

CHANNELS 6 Xposition Yposition Zposition Yrotation Xrotation Zrotation

JOINT RightHand

{

OFFSET -28.00 0.00 0.00

CHANNELS 6 Xposition Yposition Zposition Yrotation Xrotation Zrotation

JOINT RightHandThumb1

{

OFFSET -2.70 0.21 3.39

CHANNELS 6 Xposition Yposition Zposition Yrotation Xrotation Zrotation

JOINT RightHandThumb2

{

OFFSET -2.75 -0.64 2.83

CHANNELS 6 Xposition Yposition Zposition Yrotation Xrotation Zrotation

JOINT RightHandThumb3

{

OFFSET -2.13 -0.81 1.59

CHANNELS 6 Xposition Yposition Zposition Yrotation Xrotation Zrotation

End Site

{

OFFSET -1.80 -0.90 1.80

}

}

}

}

JOINT RightInHandIndex

{

OFFSET -3.50 0.55 2.15

CHANNELS 6 Xposition Yposition Zposition Yrotation Xrotation Zrotation

JOINT RightHandIndex1

{

OFFSET -5.67 -0.10 1.09

CHANNELS 6 Xposition Yposition Zposition Yrotation Xrotation Zrotation

JOINT RightHandIndex2

{

OFFSET -3.92 -0.19 0.20

CHANNELS 6 Xposition Yposition Zposition Yrotation Xrotation Zrotation

JOINT RightHandIndex3

{

OFFSET -2.22 -0.14 -0.08

CHANNELS 6 Xposition Yposition Zposition Yrotation Xrotation Zrotation

End Site

{

OFFSET -2.28 0.00 0.00

}

}

}

}

}

JOINT RightInHandMiddle

{

OFFSET -3.67 0.56 0.82

CHANNELS 6 Xposition Yposition Zposition Yrotation Xrotation Zrotation

JOINT RightHandMiddle1

{

OFFSET -5.62 -0.09 0.34

CHANNELS 6 Xposition Yposition Zposition Yrotation Xrotation Zrotation

JOINT RightHandMiddle2

{

OFFSET -4.27 -0.29 -0.20

CHANNELS 6 Xposition Yposition Zposition Yrotation Xrotation Zrotation

JOINT RightHandMiddle3

{

OFFSET -2.67 -0.21 -0.24

CHANNELS 6 Xposition Yposition Zposition Yrotation Xrotation Zrotation

End Site

{

OFFSET -2.28 0.00 0.00

}

}

}

}

}

JOINT RightInHandRing

{

OFFSET -3.65 0.59 -0.14

CHANNELS 6 Xposition Yposition Zposition Yrotation Xrotation Zrotation

JOINT RightHandRing1

{

OFFSET -5.00 -0.02 -0.52

CHANNELS 6 Xposition Yposition Zposition Yrotation Xrotation Zrotation

JOINT RightHandRing2

{

OFFSET -3.65 -0.29 -0.74

CHANNELS 6 Xposition Yposition Zposition Yrotation Xrotation Zrotation

JOINT RightHandRing3

{

OFFSET -2.55 -0.19 -0.44

CHANNELS 6 Xposition Yposition Zposition Yrotation Xrotation Zrotation

End Site

{

OFFSET -2.16 0.00 0.00

}

}

}

}

}

JOINT RightInHandPinky

{

OFFSET -3.43 0.51 -1.30

CHANNELS 6 Xposition Yposition Zposition Yrotation Xrotation Zrotation

JOINT RightHandPinky1

{

OFFSET -4.49 -0.02 -1.18

CHANNELS 6 Xposition Yposition Zposition Yrotation Xrotation Zrotation

JOINT RightHandPinky2

{

OFFSET -2.85 -0.16 -0.90

CHANNELS 6 Xposition Yposition Zposition Yrotation Xrotation Zrotation

JOINT RightHandPinky3

{

OFFSET -1.77 -0.14 -0.66

CHANNELS 6 Xposition Yposition Zposition Yrotation Xrotation Zrotation

End Site

{

OFFSET -1.68 0.00 0.00

}

}

}

}

}

}

}

}

}

JOINT LeftShoulder

{

OFFSET 3.50 8.06 0.00

CHANNELS 6 Xposition Yposition Zposition Yrotation Xrotation Zrotation

JOINT LeftArm

{

OFFSET 17.50 0.00 0.00

CHANNELS 6 Xposition Yposition Zposition Yrotation Xrotation Zrotation

JOINT LeftForeArm

{

OFFSET 29.00 0.00 0.00

CHANNELS 6 Xposition Yposition Zposition Yrotation Xrotation Zrotation

JOINT LeftHand

{

OFFSET 28.00 0.00 0.00

CHANNELS 6 Xposition Yposition Zposition Yrotation Xrotation Zrotation

JOINT LeftHandThumb1

{

OFFSET 2.70 0.21 3.39

CHANNELS 6 Xposition Yposition Zposition Yrotation Xrotation Zrotation

JOINT LeftHandThumb2

{

OFFSET 2.75 -0.64 2.83

CHANNELS 6 Xposition Yposition Zposition Yrotation Xrotation Zrotation

JOINT LeftHandThumb3

{

OFFSET 2.13 -0.81 1.59

CHANNELS 6 Xposition Yposition Zposition Yrotation Xrotation Zrotation

End Site

{

OFFSET 1.80 -0.90 1.80

}

}

}

}

JOINT LeftInHandIndex

{

OFFSET 3.50 0.55 2.15

CHANNELS 6 Xposition Yposition Zposition Yrotation Xrotation Zrotation

JOINT LeftHandIndex1

{

OFFSET 5.67 -0.10 1.08

CHANNELS 6 Xposition Yposition Zposition Yrotation Xrotation Zrotation

JOINT LeftHandIndex2

{

OFFSET 3.92 -0.19 0.20

CHANNELS 6 Xposition Yposition Zposition Yrotation Xrotation Zrotation

JOINT LeftHandIndex3

{

OFFSET 2.22 -0.14 -0.08

CHANNELS 6 Xposition Yposition Zposition Yrotation Xrotation Zrotation

End Site

{

OFFSET 2.28 0.00 0.00

}

}

}

}

}

JOINT LeftInHandMiddle

{

OFFSET 3.67 0.56 0.82

CHANNELS 6 Xposition Yposition Zposition Yrotation Xrotation Zrotation

JOINT LeftHandMiddle1

{

OFFSET 5.62 -0.09 0.34

CHANNELS 6 Xposition Yposition Zposition Yrotation Xrotation Zrotation

JOINT LeftHandMiddle2

{

OFFSET 4.27 -0.29 -0.20

CHANNELS 6 Xposition Yposition Zposition Yrotation Xrotation Zrotation

JOINT LeftHandMiddle3

{

OFFSET 2.67 -0.21 -0.24

CHANNELS 6 Xposition Yposition Zposition Yrotation Xrotation Zrotation

End Site

{

OFFSET 2.28 0.00 0.00

}

}

}

}

}

JOINT LeftInHandRing

{

OFFSET 3.65 0.59 -0.14

CHANNELS 6 Xposition Yposition Zposition Yrotation Xrotation Zrotation

JOINT LeftHandRing1

{

OFFSET 5.00 -0.02 -0.52

CHANNELS 6 Xposition Yposition Zposition Yrotation Xrotation Zrotation

JOINT LeftHandRing2

{

OFFSET 3.65 -0.29 -0.74

CHANNELS 6 Xposition Yposition Zposition Yrotation Xrotation Zrotation

JOINT LeftHandRing3

{

OFFSET 2.55 -0.19 -0.44

CHANNELS 6 Xposition Yposition Zposition Yrotation Xrotation Zrotation

End Site

{

OFFSET 2.16 0.00 0.00

}

}

}

}

}

JOINT LeftInHandPinky

{

OFFSET 3.43 0.51 -1.30

CHANNELS 6 Xposition Yposition Zposition Yrotation Xrotation Zrotation

JOINT LeftHandPinky1

{

OFFSET 4.49 -0.02 -1.18

CHANNELS 6 Xposition Yposition Zposition Yrotation Xrotation Zrotation

JOINT LeftHandPinky2

{

OFFSET 2.85 -0.16 -0.90

CHANNELS 6 Xposition Yposition Zposition Yrotation Xrotation Zrotation

JOINT LeftHandPinky3

{

OFFSET 1.77 -0.14 -0.66

CHANNELS 6 Xposition Yposition Zposition Yrotation Xrotation Zrotation

End Site

{

OFFSET 1.68 0.00 0.00

}

}

}

}

}

}

}

}

}

}

}

}

}

}

MOTION

Frames: 0

Frame Time: 0.010

Appendix C: Skeleton graph

Revision history

| Revision | Author | date | Description/changes |
| --- | --- | --- | --- |
| D1 | Yuanhui He | 12/22/2014 | Initial released |
| D2 | Peng Gao | 12/22/2014 | Added: Description of APIs. Modified: Format edit. |
| D3 | Siyuan Deng | 12/23/2014 | Added: English translation. Modified: |
| D4 | Yuanhui He | 12/25/2014 | Added: Modified: Delete string data stream type. |
| D5 | Jinzhou Chen | 12/25/2014 | Modified: Modify the English translation. |
| D6 | Yuanhui He | 12/25/2014 | Modified: Modify the English translation and some API description. |
| D7 | Yuanhui He Siyuan Deng | 1/26/2015 | Added: Appendix, Skeleton Data format Modified: Data format description |
| D8 | Yuanhui He | 2/3/2015 | Added: UDP protocol type support. |
| D9 | Tobi | 11/3/2015 | Modify: English review. |
| D10 | Yuanhui He | 20/3/2015 | Add: Multi-client is supported. Modify: English review. |
| D11 | Yuanhui He | 24/4/2015 | Add: Some commands or APIs added to be used to sync parameters or data from server. Delete: Unity demo |